Photos of examples are on the pages below.

SPIRALING WHEEL

1. Orange peel texture (pink)- cutter 12:00 handle 1:30- straight in and out.  Short fine lines (blue)- cutter 12:00 handle 12:00 straight in and out.  Extended fine lines (green)- slide to the left back and forth.

2. 1/8-inch bold lines in a ring (pink)- cutter 12:00 handle 12:00 straight in and hold in one spot.  Ring of triangles (orange)- cutter 12:00 handle 1:30 pivot into wood and hold pressing into and to the left.  Multiple rings of triangles (blue)- cutter 12:00 handle 1:30 hold onto wood lift move over a little and repeat.

3. Basic spiral- cutter 1:00 handle 1:00 touch wood and roll cutter downward toward 7:30.

4. Spiral with feather tips- cutter 1:00 handle 1:00 pull cutter straight left at end of 7:30 downward roll.

5. Spiral with feather tips using fine tooth cutter for comparison to mild tooth cutter- same technique as #4.

6. Petal rings- cutter 12:00 handle 1:30 pivot into wood and hold pressing into and to the left firmly.

7. Petals with a spiral (green)- cutter 12:00 handle 1:30 set petals then slightly rotate cutter clockwise and roll down toward 7:30.  Petals with texture (blue)- cutter 12:00 handle 1:30 set petals then drag cutter straight left.

8. Petals with long spiral (pink)- do same as #7 but roll down more .  Petals with jagged spiral (blue)- cutter 12:00 handle 1:30 set petals then pull cutter left while rotating handle clockwise.

9. Bold jagged spiral- cutter 12:00 handle 1:30 set petals then rotate cutter to 1:00 roll down toward 7:30 then pull left at end.

10. Concentric circles- cutter 3:00 handle 1:30 flat side of cutter up start on center line and roll left.  Easier with texturing wheel.

TEXTURING WHEEL

11. Orange peel (red)- cutter 12:00 handle 12:00 straight in and out.  Shallow petals (blue)- cutter 12:00 handle 2:00 pivot into wood with hard side pressure.  Clockwise spital (black)- cutter 1:00 handle 1:00 start on or above center line and roll cutter down toward 7:30.

12. Messy spiral with starry border- cutter 1:30 handle 1:30 start cut at center line roll down toward 7:30 and when almost at the end rotate cutter counterclockwise to 12:00 and make stars.

13. Huge messy spiral- cutter1:30 handle 1:30 start on center line then drag straight left multiple times.

Acknowledgement:

Burt Hovander, YouTube presentation on Spiraling and Texturing